PROMETRONICS  ·  FOR DISCUSSION WITH QUANTUM DESIGN EUROPE

Which vertical do we start with?

Seven candidate markets for an automated QCL-IR system, assessed on workflow alignment, reusable spectral libraries, which automation items carry the value, obstacles and bottom-up instrument volume

| Field | Value |
| --- | --- |
| Prepared for | Carsten Pape, Quantum Design Europe |
| From | Christian Pfalz, ProMetronics |
| Date | August 2026 |
| Purpose | To choose the launch vertical for the Spero automation project. Estimates are bottom-up with assumptions shown; QDE input replaces them where available. |
| Excluded | Microplastics as a launch market — QDE reports one client, no obligation exists, and the EU watch-list decision has not been taken. |

# 1. The recommendation, up front

Start with plastics and the chemical industry, together. Follow with pharma. They share the same buyer behaviour, the same failure-analysis workflow and — decisively — the most mature commercial IR spectral libraries in existence. There is no validation burden, so revenue can arrive in months rather than years. Pharma is the larger and more durable prize and should run behind them, not instead of them.

The reason is not market size. Pharma is bigger on every count. The reason is that plastics and chemicals are the only verticals where we can reuse existing spectral libraries almost wholesale, and where nothing regulatory stands between a working product and an invoice.

# 2. How the volume estimates were built

Market-research figures for IR spectroscopy are not usable for this decision — published 2026 values for nominally the same market range from USD 0.42 bn to 1.44 bn, a factor of 3.4. The estimates below are therefore bottom-up: an addressable laboratory population, a five-year adoption share, divided by five to give an annual unit rate.

Every input is an assumption and is shown. QDE should overwrite them where it has better data — particularly the laboratory populations, which QDE knows from its own installed base and enquiry history.

| Vertical | Labs (est.) | 5-yr adoption | Units / year | Basis of lab count |
| --- | --- | --- | --- | --- |
| Pharma / biopharma | 1,600 | 6 % | 19 | Parenteral and biologics QC sites, global |
| Chemicals | 2,400 | 3 % | 14 | Industrial QC and failure-analysis labs, EU + NA |
| Plastics & polymers | 1,800 | 3.5 % | 13 | Converter and compounder labs with a microscopy need |
| Food | 900 | 2 % | 4 | Food QC labs doing foreign-body work |
| Petrochemicals | 450 | 3 % | 3 | Refinery and lubricant laboratories |
| Cosmetics | 400 | 2.5 % | 2 | Cosmetics QC plus contract laboratories |
| Environmental | 120 | 4 % | 1 | Labs with microplastics capability or intent |
| Total |  |  | 55 |  |

Table 1 — Sanity check: today's entire QCL-IR imaging market across all vendors is of the order of 70 units per year, research-driven. A model total of 55 therefore implies that an automated system roughly doubles the category rather than taking share within it — which is only true if the industrial buyer is real. That is the central assumption of the whole project.

# 3. What we are actually automating

Taken from the ProMetronics workflow analysis of the Spero-QT plus ImageLab chain. Nine items are deliverable with today's stack; eight are the physics-informed roadmap.

|  | Deliverable today (T1–T9) | Where it applies in the workflow |
| --- | --- | --- |
| T1 | Learning ROI detection | ROI selection is manual today — the single largest unattended-operation blocker |
| T2 | AI spectral classification | Replaces per-study chemometric models |
| T3 | Scalable spectral search | Replaces similarity matching rebuilt for each study |
| T4 | Inference as a service (API) | Cross-cutting infrastructure |
| T5 | Central data lake | Replaces local files |
| T6 | MLOps and model registry | Replaces per-study models |
| T7 | Self-learning loop | QA and validation, today static expert review |
| T8 | Automatic reports | Report generation is manual today |
| T9 | PROM feature extraction | Replaces manual descriptors |

Table 2 — Two items describe manual steps outright: ROI selection (T1) and report generation (T8). Those are the two that convert directly into Time to Result.

The physics roadmap (P1–P8) is deliberately not part of this decision: scattering correction, physics-based unmixing, inverse parameter determination, sparse acquisition, process state estimation, uncertainty quantification, cross-instrument transfer and the spectroscopy physics template. These map onto the Leonardo roadmap discussion, not onto the launch vertical.

One structural point worth carrying into the QDE conversation: the T-items split into a throughput cluster (T1, T2, T3, T8, T9) and an infrastructure cluster (T4, T5, T6, T7). Small industrial laboratories buy the first. Large regulated organisations buy both, and value the second more. That split largely explains the ranking below.

# 4. The seven verticals

## 3.1  Plastics and polymers    —    13 units / year

Workflow alignment.  Internal incoming-goods verification, batch release, customer-complaint and failure-analysis reports. Structured but not externally audited; ISO 9001 or 17025 in the better labs. Automotive suppliers additionally follow VDA documentation practice. This is workflow we can align to quickly because the report is internal — we are not constrained by a normative template.

Reusable IR libraries.  The best of any vertical. Polymer identification is the most mature commercial IR library domain that exists — Bio-Rad KnowItAll, S.T. Japan, and the vendor libraries from Bruker and Thermo all cover it deeply. On the open side: FLOPP and FLOPP-e (µATR-FTIR, incl. weathered material), Open Specy (>30,000 spectra, CC-BY-4.0) and Primpke's QCL-measured database of 289 polymers. Very little new library work required.

Which automation items carry the value.  T2 and T3 carry most of the value — AI classification and scalable spectral search, both of which rest on library quality, which is highest here. T8 (automatic reports) removes the manual reporting step. T1 matters less: in defect analysis the operator usually already knows where to look.

Obstacles.  Bulk ATR-FTIR already solves routine polymer identification cheaply, so the microscopy case is narrower than the lab population suggests: multilayer films, small defects, inclusions, contamination. We must sell into that slice, not into polymer ID generally.

## 3.2  Chemical industry    —    14 units / year

Workflow alignment.  Internal SOPs, incoming goods inspection, batch release, deviation and complaint handling. ASTM E168 and E1252 cover general IR practice. Highly variable between companies, which cuts both ways: no template to satisfy, but also no template to design against. Failure analysis is the sharpest entry point because it carries urgency.

Reusable IR libraries.  Excellent. The large commercial libraries were built for exactly this market — solvents, additives, contaminants, raw materials. Reuse is close to complete for identification; our work is in orchestration, statistics and reporting rather than in spectroscopy.

Which automation items carry the value.  As for plastics, plus T7 (self-learning loop), which pays where the same contaminant types recur across cases.

Obstacles.  No regulatory forcing function at all. The sale is pure efficiency, which means it depends entirely on Time to Result being quantified in a customer's own numbers. Buyer is fragmented across many sites and companies.

## 3.3  Pharma and biopharma    —    19 units / year

Workflow alignment.  The most structured of all — and structure helps as much as it hinders. USP ⟨788⟩/⟨789⟩ and Ph. Eur. 2.9.19/2.9.20 make subvisible particle counting mandatory release testing on every parenteral batch; USP ⟨787⟩ covers therapeutic proteins; USP ⟨1787⟩ gives identification guidance. EU GMP Annex 1 (2022) requires a documented Contamination Control Strategy, which is a direct driver for particle source identification.

Reusable IR libraries.  Good. Pharmacopoeial reference spectra exist, and commercial excipient, API and contaminant libraries are mature. Particulate contamination libraries (glass, silicone oil, elastomer, polymer, protein aggregate) are available commercially.

Which automation items carry the value.  The infrastructure cluster is the differentiator here — T4 (inference API), T5 (central data lake) and T6 (MLOps and model registry) replace what your own analysis calls today's state: local files, per-study models, single workstation. T8 serves GMP documentation, and the benefit dimension your analysis labels 'trustworthy, decision-grade' names pharma explicitly. P6 (uncertainty quantification) is what a regulated decision eventually needs.

Obstacles.  The decisive one: GMP, 21 CFR Part 11, GAMP 5 and computerised system validation put 12–18 months between a working product and first revenue. Change control makes every subsequent update expensive. Also note the split: counting is mandatory, identification is guidance — so our use case sits in investigations, not in per-batch routine.

## 3.4  Food    —    4 units / year

Workflow alignment.  HACCP, IFS and BRC audit documentation, ISO 22000 or FSSC 22000. Well structured and externally audited. The relevant use case is foreign-body and contaminant identification following a customer complaint — which is urgent, documented, and currently slow.

Reusable IR libraries.  Moderate. Packaging and food-contact material libraries exist commercially; foreign-body contaminant libraries are thinner and often built in-house by the testing lab.

Which automation items carry the value.  T1 is the key item — in foreign-body work nobody knows where the particle is, so automated ROI detection is the whole job. T8 supports complaint documentation.

Obstacles.  Migration testing under EU 10/2011 is chromatography, not IR, so the largest regulated analytical workload in food is not ours. Authenticity testing is dominated by NIR, NMR and isotope methods. The addressable slice is foreign bodies only.

## 3.5  Petrochemicals    —    3 units / year

Workflow alignment.  ASTM-heavy and highly disciplined, but the ASTM methods that matter are bulk — fuels, lubricants, additives. Refinery and lubricant labs run tightly controlled SOPs. Deposit, filter and catalyst analysis is where microscopy enters, and that work is usually internal and undocumented externally.

Reusable IR libraries.  Moderate. Petroleum and lubricant libraries exist; deposit and fouling libraries are thin and largely proprietary to the operator.

Which automation items carry the value.  T2 and T3, at low volume.

Obstacles.  The analytical workhorses here are GC, GC×GC, ICP and NMR. IR microscopy is genuinely niche. Small number of sites, though budgets per site are high and sales cycles are long.

## 3.6  Cosmetics    —    2 units / year

Workflow alignment.  EU Cosmetics Regulation 1223/2009 with the Product Information File and safety assessment; ISO 22716 as GMP. Structured, lighter than pharma. Raw-material identity and contamination are the IR use cases.

Reusable IR libraries.  Moderate. Cosmetic raw-material libraries exist commercially.

Which automation items carry the value.  T2 and T3, at low volume.

Obstacles.  We had hoped the REACH microplastics restriction (Entry 78, in force since October 2023) would provide a regulatory hook. It does not: the threshold is 0.01 % by weight and the temporary lower size limit is 0.1 µm, so the method in practice is pyrolysis-GC/MS, and Spero's ~5 µm diffraction limit is fifty times too coarse. The Commission's own guidance prescribes no method at all. This vertical carries a regulation that does not create demand for our instrument.

## 3.7  Environmental analytics    —    1 unit / year

Workflow alignment.  EN ISO 16094-2:2025 is the best-written standard for our purposes anywhere — it prescribes microscopy with vibrational spectroscopy and explicitly anticipates automated analysis of large particle numbers. ISO 17025 accreditation governs the labs.

Reusable IR libraries.  Excellent and open: Open Specy, FLOPP/FLOPP-e, siMPle and Primpke's QCL database — all free or open access, and Primpke's set was measured on a Spero.

Which automation items carry the value.  Technically the best fit for the full T-set — T1 over a whole filter with thousands of unknown particle positions is precisely what learning ROI detection is for. Best showcase, absent market.

Obstacles.  Nobody is obliged to run it. Microplastics has not been added to the EU drinking-water watch list; the full risk assessment is due 2029; Germany has no systematic monitoring and no federal data (Bundestag Drs. 21/5463, April 2026). QDE reports a single client. Excellent method fit, absent market.

# 5. Comparison

| Vertical | Units / yr | Library reuse | Regulatory drag | Verdict |
| --- | --- | --- | --- | --- |
| Plastics & polymers | 13 | Highest | None | Launch |
| Chemicals | 14 | Very high | None | Launch |
| Pharma / biopharma | 19 | High | Severe — 12–18 mo | Follow |
| Food | 4 | Moderate | Low | Opportunistic |
| Petrochemicals | 3 | Moderate | Low | Opportunistic |
| Cosmetics | 2 | Moderate | Regulation exists but points elsewhere | Not now |
| Environmental | 1 | Highest, open | None — and no obligation either | Reference only |

Table 2 — Note that units per year and launch suitability rank differently. Pharma leads on volume and comes last on speed.

# 6. Why plastics and chemicals together

- They are one buyer, not two. A compounder's failure-analysis lab and a chemical company's contamination lab run the same workflow on the same instrument with the same urgency. One product serves both.

- Library work is close to zero. This directly answers the question of what the identification layer costs us: for these two verticals, less than for any other.

- The use case genuinely needs microscopy. Multilayer films, inclusions, small defects and surface contamination cannot be done with bulk ATR — so Spero is the right instrument rather than an expensive alternative to a cheap one.

- Failure analysis has a clock on it. A line stoppage or a customer complaint costs money per hour. That is where Time to Result converts into a number the customer can put in a business case — and it is the only vertical where we can price against a loss rather than against a salary.

- QDE already has the accounts. Industrial customers, not research groups.

# 7. What we need from QDE

| # | Question | Why it matters |
| --- | --- | --- |
| 1 | How many enquiries per vertical in the last 24 months, and from what kind of organisation? | Replaces our adoption assumptions with observed demand. |
| 2 | How many IR microscopes has QDE placed, by vertical? | Calibrates the laboratory populations in Table 1. |
| 3 | In plastics and chemicals, what does a customer currently pay per failure-analysis case, and how long does it take? | Turns Time to Result into a euro figure. Without it we cannot price. |
| 4 | Which three customers would take a pilot, and would they co-develop rather than evaluate? | Determines whether the pilot phase can start immediately. |
| 5 | Does QDE reach pyrolysis-GC/MS buyers as well as IR buyers? | Relevant only if we later revisit the cosmetics or microplastics regulation route. |

Now incorporated: the ProMetronics workflow analysis. Section 3 reflects the T1–T9 and P1–P8 items from FTIR_Workflow_Improvement_Potentials (9 July 2026), including the two workflow steps identified there as manual today — ROI selection and report generation. The per-vertical assessments in Section 4 name which items carry the value in each market.

Sources: EN ISO 16094-2:2025 · DIN EN ISO 24187 · Commission Delegated Decision on microplastics methodology (2024) · Bundestag Drucksache 21/5463 (20 April 2026) · Commission Explanatory Guide to REACH Annex XVII entry 78 · Regulation (EU) 2023/2055 · USP ⟨787⟩/⟨788⟩/⟨789⟩/⟨1787⟩, Ph. Eur. 2.9.19/2.9.20 · EU GMP Annex 1 (2022) · Open Specy (Cowger et al., Anal. Chem. 2021) · FLOPP/FLOPP-e (Anal. Chem. 2021) · Primpke, Godejohann & Gerdts, Environ. Sci. Technol. 2020.